# Prognostics and Health Management (PHM) Failure Modes Catalog

## 14-Profile Localized Degradation & System Propagation Specification

This catalog provides an exhaustive technical reference for the 14 localized robotic failure profiles and the propagation shock mode implemented in the Fault Registry. It serves to guide developers and data scientists in understanding the physical phenomena, joint eligibilities, mathematical profiles, and industrial signatures of each failure.

graph TD
    A[PHM Failure Registry] --> B[Tier A: Physics-Informed Full]
    A --> C[Tier B: Structural Compact]
    A --> D[System Propagation Shock]

    B --> B1[8 Profiles: BL, LF, BW, EE, MD, ED]
    C --> C1[6 Profiles: BL, LF, BW, EE, MD, ED]
    D --> D1[1 Profile: SYS_COLLISION]

## 1. Complete Failure Registry Reference Table

| Profile ID | Fault Key | Failure Mode Name | Eligible Joints | Severity Curve | Model Type | Physical Rationale / Signature |
| --- | --- | --- | --- | --- | --- | --- |
| BL_FULL | FAULT_BL | Gear Backlash | 0, 1, 2 | Exponential | Full | Cycloidal/harmonic gear dead-zone at base, shoulder, elbow. |
| LF_DIST_FULL | FAULT_LF | Distributed Lubrication Failure | All (0–5) | Exponential | Full | System-wide boundary lubrication transition with load weighting. |
| LF_JOINT_FULL | FAULT_LF | Localized Lubrication Failure | All (0–5) | Exponential | Full | Single-joint viscous friction escalation before propagation. |
| BW_FULL | FAULT_BW | Bearing Wear RCF | 0, 1, 4, 5 | Linear | Full | MCSA sidebands and spall pulses on high-load/speed joints. |
| EE_FULL | FAULT_EE | Electrical Erosion EDM | All (0–5) | Linear | Full | Parasitic bearing currents and step-wise thermal fluting. |
| MD_INC_FULL | FAULT_MD | Motor Demagnetization — Incipient | 1, 2 | Exponential | Full | Gradual torque constant (Kt) decay under holding torque. |
| MD_ABR_FULL | FAULT_MD | Motor Demagnetization — Abrupt | 1, 2 | Exponential | Full | Step-drop Kt from thermal overload or short circuit. |
| ED_FULL | FAULT_ED | Encoder Drift | All (0–5) | Exponential | Full | Cumulative feedback bias, phantom PID jitter, error latching. |
| BL_COMPACT | FAULT_BL | Gear Backlash | 0, 1, 2 | Exponential | Compact | Dead-zone lag and re-engagement strikes (reduced channels). |
| LF_COMPACT | FAULT_LF | Distributed Lubrication Failure | All (0–5) | Linear | Compact | Viscous drag and thermal ramp across all six joints. |
| BW_COMPACT | FAULT_BW | Bearing Wear | 0, 1, 4, 5 | Polynomial | Compact | RCF velocity jitter and torque ripple on motor current. |
| EE_COMPACT | FAULT_EE | Electrical Erosion | All (0–5) | Abrupt | Compact | Arcing current spikes and localized thermal steps. |
| MD_COMPACT | FAULT_MD | Motor Demagnetization — Incipient | 1, 2 | Linear | Compact | Kt decay with inverse current elevation (compact model). |
| ED_COMPACT | FAULT_ED | Encoder Drift | All (0–5) | Linear | Compact | Linear measurement bias added to actual joint position. |
| SYS_COLLISION | FAULT_SYS | Payload / Collision Shock | All (0–5) | Constant | Full | Half-sine impulse and damped oscillation on motor currents. |

## 2. Technical Deep-Dives: The 6 Root Failure Modes

### 1. Gear Backlash (FAULT_BL)

Backlash represents the play or clearance between mating gear teeth in joint gearboxes (cycloidal, harmonic, or planetary). * Physical Cause: Abrasive wear of gear teeth or loss of pre-tension, causing a dead-zone when the motor reverses direction. * ML Signature: - In Position: Dead-band lag where motor position changes but joint output stays static during direction changes. - In Current/Torque: Re-engagement strikes (sharp, high-frequency torque spikes when teeth reconnect). * Eligible Joints (0, 1, 2): Restricted to base (Joint 0), shoulder (Joint 1), and elbow (Joint 2) due to high gravity/inertial payloads which experience severe mechanical impacts. * Difference (Full vs. Compact): BL_FULL models full kinematic coupling (phase delays and impacts across multiple state channels), whereas BL_COMPACT directly injects positional backlash lag with reduced mathematical channels.

### 2. Lubrication Failure (FAULT_LF)

Lubrication failure simulates the degradation of synthetic grease in joint bearings and gearboxes. * Physical Cause: Grease dry-out, thermal breakdown, oil-soap separation, or dust contamination, leading to a transition from hydrodynamic (fluid film) lubrication to dry boundary lubrication. * ML Signature: - Distributed (LF_DIST_FULL): Exponential system-wide friction escalation. High gravity-loaded joints experience larger friction steps. - Localized (LF_JOINT_FULL): Viscous friction spikes on a targeted joint, followed by thermal propagation across adjacent channels. * Eligible Joints (All 0–5): All joints are eligible since all contain physical bearings and grease. * Difference (Full vs. Compact): The Full tier uses physics-based multi-stage friction equations (viscous, static, and Stribeck curve dynamics). The Compact tier uses simple linear scaling of drag force.

### 3. Bearing Wear RCF (FAULT_BW)

Bearing Wear models Rolling Contact Fatigue (RCF) on the inner/outer raceways or balls of joint support bearings. * Physical Cause: Sub-surface crack propagation caused by cyclic stress, leading to microscopic spalls (pits or craters on steel surfaces). * ML Signature: - In Frequency Domain (MCSA): Induction of sideband frequencies on the motor current. - In Time Domain: Periodic, high-frequency impact pulses occurring at bearing defect frequencies (BPFO/BPFI). * Eligible Joints (0, 1, 4, 5): Restricted to Joints 0 (base), 1 (shoulder), 4, and 5 (wrist joints) because they sustain the highest radial loads and high-speed wrist articulations. * Difference (Full vs. Compact): BW_FULL models high-fidelity spall pulses overlapping motor currents, while BW_COMPACT injects high-frequency velocity jitter and current noise.

### 4. Electrical Erosion EDM (FAULT_EE)

Electrical Erosion simulates the degradation of bearings due to unwanted electrical currents. * Physical Cause: Inverter-driven variable frequency drives (VFD) induce parasitic high-frequency shaft voltages. When these exceed the bearing grease film insulation limit, they discharge through the balls, creating electric sparks (Electrical Discharge Machining - EDM). * ML Signature: - Thermal Fluting: Distinct micro-grooves (“flutes”) on raceways causing stepping torque resistance. - In Current: High-frequency, discontinuous arcing spikes. * Eligible Joints (All 0–5): All joints are motor-driven via individual VFD controllers and are subject to shaft voltage buildup. * Difference (Full vs. Compact): EE_FULL models structural fluting with high thermal stepping functions, while EE_COMPACT injects abrupt arcing noise spikes in joint torques.

### 5. Motor Demagnetization (FAULT_MD)

Demagnetization models the permanent degradation of the rare-earth magnets inside the robot’s synchronous servomotors. * Physical Cause: Thermal overload (overheating due to holding excessive gravity payloads for too long) or electrical short-circuits. * ML Signature: - Loss of Kt (Torque Constant): The motor must draw significantly higher current/torque commands to produce the exact same mechanical torque. - Efficiency Drop: Dramatic heat generation and stator winding temperature spikes. * Eligible Joints (1, 2): Exclusively restricted to the shoulder (Joint 1) and elbow (Joint 2), as they are the primary load-bearing joints that work continuously against gravity to hold up the arm payload. * Difference (Incipient vs. Abrupt): - Incipient (MD_INC_FULL): Gradual decay of torque capabilities, modeling magnet temperature climbing slowly. - Abrupt (MD_ABR_FULL): A sudden, massive step-drop in torque output, simulating a coil short-circuit or massive demagnetization overload. - Compact (MD_COMPACT): Linear decay in torque efficiency with simplified current feedback loops.

### 6. Encoder Drift (FAULT_ED)

Encoder Drift models the progressive degradation of joint optical or magnetic encoders. * Physical Cause: Thermal warping, dust/grease contamination on the optical code disk, or cumulative signal cable fatigue. * ML Signature: - Cumulative Bias: Positional reading gradually drifts away from actual joint coordinates. - PID Compensation Jitter: The control loop attempts to correct for this phantom displacement, causing high-frequency control loop oscillations and physical jitter. * Eligible Joints (All 0–5): Every joint relies on closed-loop feedback encoders. * Difference (Full vs. Compact): ED_FULL implements error latching and cumulative PID loop feedback instability. ED_COMPACT directly adds a linear sensor offset to the positional signal.

## 3. Propagation & System-Level Shock (SYS_COLLISION)

Unlike the 14 localized profiles, SYS_COLLISION is a system-wide, propagation-level event: * Physical Cause: Robot collisions with structural barriers, dropping heavy payloads, or rapid mechanical jams. * ML Signature: A sharp half-sine acceleration impulse, followed by decaying damped harmonic oscillation waveforms across all motor currents, joint velocities, and torques. * Eligible Joints (All 0–5): Impacts physically propagate throughout the entire robotic structure, instantly impacting all six axes simultaneously. * Severity Profile (constant): The severity stays flat at 1.0 during the shock to signify that the impact occurred at maximum energy.